JET ENGINE ECU

Technical Design & Requirements Document

Version 1.0

Prepared by:  National Aerospace Science & Technology Park (NASTP)

Project:  JET Engine Ground Test Control Unit

Date:  June 2026

DOCUMENT PURPOSE

This document defines the functional requirements, hardware design, pin allocation, circuit topology, firmware architecture, and GUI specification for the JET Engine ECU project. It is written to be implementation-independent so that the system can be reproduced or extended by any developer using any compatible toolchain.

1.  Project Overview

The JET Engine ECU is a custom-designed Engine Control Unit intended to monitor and control a small gas turbine (JET) engine during ground testing. The system consists of an embedded microcontroller board connected to engine sensors and actuators, linked by a serial data link to a PC-based graphical dashboard for operator monitoring and control.

The project is structured in two major components:

Embedded Firmware — runs on the microcontroller, reads all sensors, controls all actuators, and streams telemetry over a UART serial link at 115,200 baud.

GUI Ground Station — a desktop application that displays all telemetry channels in real time, allows the operator to toggle engine actuators (FET channels), and logs all incoming serial data.

1.1  Design Goals

All sensor readings updated independently and continuously without blocking the main execution loop.

Serial link is bidirectional — firmware streams data out, GUI sends control commands in over the same wire.

GUI always reflects actual hardware state, not just the last user interaction.

All actuator outputs default to a safe (OFF) state at power-up before any other code runs.

The design is modular — individual sensor channels can be added or removed without restructuring the whole firmware.

1.2  Scope — Current Implementation

Two thermocouple channels: exhaust gas temperature (EGT) and intake/ambient temperature.

Battery voltage monitoring with resistor-divider scaling and calibration.

Engine RPM — simulated via potentiometer for ground testing; interrupt-based real measurement is pending.

Six FET output channels with bidirectional real-time GUI control.

Microcontroller internal die temperature monitoring.

Full PC-based dashboard GUI with circular gauges, thermometer bars, FET panel, and serial log.

2.  System Requirements

The following table defines all functional requirements for the complete system. These requirements govern both the embedded firmware and the GUI ground station.

| # | Requirement | Details |
| --- | --- | --- |
| R1 | Dual thermocouple reading | Read two K-type thermocouples via dedicated amplifier ICs. TC1 for exhaust gas (EGT), TC2 for intake/ambient. Must handle open-circuit faults gracefully (return 0°C on fault). |
| R2 | Battery voltage monitoring | Monitor main battery pack (12V–24V) using a resistor voltage divider scaled to ADC reference. Calibration applied to correct gain error. Accuracy ±0.1V across full range. |
| R3 | RPM measurement | Measure engine RPM from a conditioned digital pulse signal. Support up to 120,000 RPM. Interrupt-based counting required for accuracy. |
| R4 | RPM simulation — test mode | During ground testing without a running engine, simulate RPM via an analog potentiometer input mapped linearly to the full 0–120,000 RPM range. |
| R5 | FET output control (6 channels) | Drive up to 6 power FET channels (fuel valve, igniter, starter relay, etc.) from digital output pins. All outputs must default LOW (safe) at power-up. |
| R6 | Bidirectional serial telemetry | Stream all sensor readings to the ground station over UART at 115,200 baud, 8N1. Accept FET toggle commands over the same link. |
| R7 | MCU die temperature | Read the on-chip die temperature sensor for thermal health monitoring of the microcontroller. Report over serial every 1 second. |
| R8 | Non-blocking firmware | All periodic tasks must be timer-driven. No blocking delays in the main loop. Each channel must update at its own independent rate. |
| R9 | GUI ground station | Desktop dashboard displaying all sensor channels in real time. Allows FET control. Logs all serial data with a clear function. Dark cockpit theme. |
| R10 | GUI–firmware FET synchronisation | The GUI must reflect actual FET hardware state — not just last button press — so firmware-side state changes are visible within 200 ms. |
| R11 | Safe disconnect behaviour | All GUI gauges must reset to zero when the serial link is disconnected, preventing stale readings from being misread as live data. |

3.  Hardware Overview

3.1  Microcontroller

The system uses an STM32F103C8T6 microcontroller (Blue Pill development board). Key characteristics relevant to this design:

ARM Cortex-M3 core at 72 MHz.

12-bit successive-approximation ADC with 16 multiplexed input channels, including one internal temperature sensor channel (channel 16).

Multiple hardware UART peripherals. UART2 (PA2=TX, PA3=RX) is used for serial telemetry.

3.3V I/O voltage. All external signals must be within this range or level-shifted.

64 KB Flash, 20 KB RAM — sufficient for this firmware.

3.2  External Components

| Component | Part / Value | Purpose |
| --- | --- | --- |
| Microcontroller | STM32F103C8T6 | 72 MHz Cortex-M3, 12-bit ADC, hardware UART, 3.3V I/O |
| Thermocouple Amp #1 | MAX31855 | K-type thermocouple interface with cold-junction compensation, SPI output, ±2°C accuracy |
| Thermocouple Amp #2 | MAX31855 | Second thermocouple channel (intake / ambient temperature) |
| Battery voltage divider | 39 kΩ + 5.6 kΩ | Scales battery voltage (up to ~24V) to MCU ADC range (0–3.3V) |
| RPM signal conditioner | Dual LM358 op-amp | Amplifies and cleans engine sensor pulses to 0V/3.3V digital edges for MCU input |
| RPM test buffer | Single LM358 (unity gain) | Isolates potentiometer output from ADC input impedance during RPM simulation |
| USB–UART adapter | 3.3V TTL type | Connects UART2 (PA2/PA3) to PC USB port for telemetry and commands |
| FET driver stage | Gate driver + power MOSFET | Drives high-current engine actuators from 3.3V logic output pins |

4.  Pin Mapping

The table below lists every allocated I/O pin. Unallocated pins are left in the default reset state (floating input with no pull). All pins operate at 3.3V logic.

| Function | Pin | Direction | Mode | Description |
| --- | --- | --- | --- | --- |
| UART TX | PA2 | Output | Push-Pull | Serial transmit — telemetry to ground station |
| UART RX | PA3 | Input | Pull-Up | Serial receive — commands from ground station |
| Battery Voltage | PA1 | Analog In | High-Z | Voltage divider output (39 kΩ upper / 5.6 kΩ lower) |
| RPM Test (Pot) | PA0 | Analog In | High-Z | Op-amp buffered potentiometer — RPM simulation (test only) |
| RPM Sensor | PB11 | Digital In | Pull-Down | LM358-conditioned pulse train from engine RPM sensor |
| TC1 Clock | PA5 | Output | Push-Pull | MAX31855 #1 SPI clock — Exhaust / EGT thermocouple |
| TC1 Data | PA6 | Input | High-Z | MAX31855 #1 SPI MISO (data from chip) |
| TC1 CS | PB0 | Output | Push-Pull | MAX31855 #1 chip select (active LOW) |
| TC2 Clock | PB13 | Output | Push-Pull | MAX31855 #2 SPI clock — Intake / Ambient thermocouple |
| TC2 Data | PB14 | Input | High-Z | MAX31855 #2 SPI MISO (data from chip) |
| TC2 CS | PB12 | Output | Push-Pull | MAX31855 #2 chip select (active LOW) |
| FET 1 | PB6 | Output | Push-Pull | Engine actuator / valve output 1 — safe state LOW |
| FET 2 | PB5 | Output | Push-Pull | Engine actuator / valve output 2 — safe state LOW |
| FET 3 | PB9 | Output | Push-Pull | Engine actuator / valve output 3 — safe state LOW |
| FET 4 | PB8 | Output | Push-Pull | Engine actuator / valve output 4 — safe state LOW |
| FET 5 | PB4 | Output | Push-Pull | Engine actuator / valve output 5 — safe state LOW |
| FET 6 | PB7 | Output | Push-Pull | Engine actuator / valve output 6 — safe state LOW |
| MCU Temperature | Internal | Analog In | ADC ch.16 | On-chip die temperature sensor — no external pin connection |

PA0 is currently shared between the future SBUS input and the RPM simulation potentiometer. It must be reassigned to a different pin before SBUS decoding is implemented.

The MCU internal temperature sensor (ADC ch.16) must be enabled in ADC configuration at startup. It does not map to any physical header pin.

5.  Circuit Design Overview

5.1  Battery Voltage Measurement

Topology: Resistor voltage divider. Upper resistor (R1 = 39 kΩ) connects battery positive to the ADC pin. Lower resistor (R2 = 5.6 kΩ) connects the ADC pin to GND.

Maximum safe input: Approximately 24.4 V before divided voltage exceeds 3.3 V. Operating range is 12V–24V.

Transfer function: V_ADC = V_battery × R2 ÷ (R1 + R2)  =  V_battery × 5600 ÷ 44600  ≈  V_battery × 0.1255

Reconstruction: V_battery = V_ADC × (R1+R2) ÷ R2 × CAL_FACTOR.  CAL_FACTOR ≈ 0.9905 corrects a ~1% gain error caused by VDDA tolerance and resistor tolerance stacking.

Noise reduction: 64 ADC samples are averaged with a short inter-sample settling delay. Result accuracy: ±0.05V across the full operating range.

5.2  Thermocouple Interface

Amplifier IC: MAX31855 — dedicated cold-junction compensated thermocouple-to-digital converter. One IC per thermocouple channel.

Interface: 3-wire SPI (Clock, CS, Data-Out). The MCU acts as SPI master in read-only mode. No MOSI line is needed.

TC1 — Exhaust / EGT: High-temperature K-type thermocouple in the engine exhaust path. Relevant range: 0°C – 900°C.

TC2 — Intake / Ambient: K-type thermocouple at engine intake. Relevant range: 0°C – 100°C.

Fault handling: The MAX31855 reports open-circuit and short faults. When a fault is detected, the firmware substitutes 0.0°C to prevent invalid display values.

Minimum read interval: 250 ms. The MAX31855 requires this time for its internal conversion. Reading faster returns stale data without any error indication.

5.3  RPM Sensing

5.3.1  Real RPM — PB11  (Pending Implementation)

Sensor: Hall-effect or optical pulse sensor generating one or more pulses per shaft revolution.

Signal conditioning: Dual LM358 op-amp circuit amplifies and limits the sensor signal to a clean 0V/3.3V digital square wave.

Measurement method: External interrupt on rising edge counts pulses. RPM = (pulse_count ÷ pulses_per_revolution) × (60,000 ÷ measurement_window_ms).

5.3.2  RPM Test / Simulation — PA0

Purpose: Allows the full RPM telemetry chain and GUI display to be verified without a running engine.

Circuit: Potentiometer wiper → unity-gain LM358 buffer → PA0 analog input. The buffer prevents potentiometer loading and protects the ADC input.

Mapping: 12-bit ADC reading (0–4095) mapped linearly to 0–120,000 RPM. Full counter-clockwise = 0 RPM, full clockwise = 120,000 RPM.

5.4  FET Output Stage

Purpose: Switch high-current engine actuators (fuel valves, igniter, starter relay) from 3.3V MCU logic outputs.

Topology: Each MCU output pin drives a gate driver stage (or small-signal transistor) whose output switches the gate of a power MOSFET. The MOSFET carries the actuator load current.

Safe state: All 6 FET output pins are configured and driven LOW as the first operation in the startup sequence, before any sensor or communication initialisation.

5.5  MCU Internal Temperature Sensor

Sensor: STM32F103 contains an internal analogue temperature sensor on ADC channel 16. It measures the die temperature of the chip, not ambient air.

Conversion formula: T (°C)  =  ((V25 − V_measured) ÷ 0.0043) + 25,  where V25 is the sensor output voltage at 25°C and 0.0043 V/°C is the average slope (from datasheet).

Chip-to-chip variation: The datasheet specifies V25 = 1.43V (typical) with ±0.15V variation. This unit has been empirically calibrated to V25 = 1.58V, giving ~24°C at room temperature. Any replacement chip may need recalibration of this constant.

5.6  Serial Communication

Port: UART2 on PA2 (TX) and PA3 (RX), connected to a USB–TTL adapter.

Settings: 115,200 baud, 8 data bits, no parity, 1 stop bit (8N1).

Common GND requirement: The USB–TTL adapter GND must be connected to the MCU board GND. Without a shared ground the UART threshold levels are undefined and communication fails.

6.  Firmware Architecture

6.1  Non-Blocking Execution Loop

The firmware uses a strictly non-blocking architecture. The main loop runs as fast as possible without fixed delays. Each periodic task maintains its own timestamp of when it last ran and executes only when enough time has elapsed. The serial command handler runs on every loop iteration to minimise command response latency.

| Task | Period | Notes |
| --- | --- | --- |
| Serial command receive | Every loop pass | Character-by-character buffering; command processed on newline receipt |
| Thermocouple read | 250 ms | Maximum rate of MAX31855; polling faster yields stale data |
| Battery voltage read | 500 ms | 64-sample ADC average; ~13 ms total acquisition time per reading |
| FET state broadcast | 200 ms | All 6 hardware FET states sent as one line; GUI updates from this |
| RPM read — test mode | 100 ms | 64-sample average of potentiometer input on PA0 |
| MCU temperature | 1000 ms | Die temperature changes slowly; 1 Hz update rate is sufficient |

6.2  ADC Acquisition Strategy

Sampling: 64 samples are taken per ADC reading. A short settling delay between samples allows the ADC input capacitor to fully charge after each sample-and-hold phase. The integer average of all 64 samples is returned.

Resolution: The ADC is operated in 12-bit mode (values 0–4095). This must be explicitly enabled at startup — many frameworks default to 10-bit (0–1023), which would cause a ×4 scale error in all calculations.

Reference: 3.3V VDDA. All voltage calculations assume this reference. Deviations in VDDA introduce a proportional gain error; the battery voltage channel uses a calibration factor to correct this.

6.3  Serial Command Parsing

Incoming bytes are read one character at a time into a string buffer. On receipt of a newline character, the complete line is evaluated. Carriage return characters are silently discarded. If the line matches the FET command format (e.g. FET3:1), the corresponding output pin is driven HIGH or LOW. All other lines are discarded. This approach is robust to malformed or unexpected input.

6.4  FET State Synchronisation

Every 200 ms the firmware reads the actual logic level of all 6 FET output pins and transmits their combined state over UART as a comma-separated list. This broadcast reflects true hardware state regardless of the cause of the last change. The GUI receives this broadcast and updates all 6 status indicators, making the display self-correcting and always consistent with hardware without any additional acknowledgement protocol.

7.  Serial Interface Specification

7.1  Physical Layer

Baud rate: 115,200

Frame format: 8N1 — 8 data bits, no parity, 1 stop bit

Voltage levels: 3.3V TTL. A level-shifting adapter is required if the host PC uses 5V logic or RS-232 levels.

Ground reference: The GND of the USB–UART adapter must be physically connected to the GND rail of the MCU board.

7.2  Firmware Output  (Firmware → PC)

All output lines are ASCII text terminated with carriage return + newline (\r\n).

| Line Format | Interval | Description |
| --- | --- | --- |
| TC1: {°C} ¦ TC2: {°C} | 250 ms | Both thermocouple readings on a single line |
| Raw: {n} V_ADC: {V} V_BATT: {V} | 500 ms | ADC raw count, voltage at pin, calculated battery voltage |
| RPM: {value} | 100 ms | Engine RPM (actual interrupt-counted or potentiometer-simulated) |
| MCU: {°C} | 1000 ms | Microcontroller die temperature |
| FET:b,b,b,b,b,b | 200 ms | State of all 6 FET outputs (1=HIGH active, 0=LOW) |
| JET ECU Ready | Once on boot | Startup confirmation message |
| TC1 error - check wiring | Once on boot if fault | Emitted only if a thermocouple IC fails initialisation |

7.3  PC Input Commands  (PC → Firmware)

Commands are ASCII strings terminated with a newline (\n). Carriage return (\r) is optional and ignored by the firmware.

| Command | Effect |
| --- | --- |
| FETn:1 (n = 1 to 6) | Drive the specified FET output HIGH. Example: FET3:1 activates channel 3. |
| FETn:0 (n = 1 to 6) | Drive the specified FET output LOW. Example: FET3:0 deactivates channel 3. |

The FET state broadcast line (FET:...) is not echoed to the GUI serial log. At 5 Hz it would otherwise dominate the log with status noise. Only actionable sensor data lines are displayed.

8.  GUI Ground Station

8.1  Technology Stack

Language: Python 3

GUI framework: tkinter (standard library, no additional installation required)

Serial communication: pyserial

Threading: standard threading module for background serial reader

8.2  Architecture

The serial port is read on a dedicated background daemon thread. This prevents the GUI from freezing during serial wait states. The background thread schedules all UI updates on the main thread using an event queue callback. No direct widget access is ever made from the background thread, ensuring thread safety without locks.

Window is fixed-size, landscape-oriented, with a dark cockpit colour theme for readability in outdoor or low-light testing environments.

8.3  Display Widgets

| Widget | Data Source | Range | Colour Zones |
| --- | --- | --- | --- |
| Tachometer — circular gauge | RPM: line | 0 – 120,000 RPM | Green 0–80k / Yellow 80–100k / Red 100–120k |
| Battery — circular gauge | V_BATT value | 0 – 26 V | Red 0–14V / Yellow 14–18V / Green 18–26V |
| TC1 — vertical thermometer | TC1: value | 0 – 900 °C | Blue → Green → Yellow → Orange → Red gradient |
| TC2 — vertical thermometer | TC2: value | 0 – 100 °C | Same gradient, compressed range |
| MCU — vertical thermometer | MCU: value | -20 – 100 °C | Warn at 70°C shown in orange / Critical 85°C in red |
| FET panel — 2×3 grid | FET: line | HIGH / LOW per channel | Green LED = HIGH active / Red LED = LOW / TOGGLE button |
| Serial log | All text lines | — | Scrolling display; CLEAR button; FET state lines excluded |

8.4  FET Panel Behaviour

Each FET channel shows a coloured LED indicator (green = HIGH, red = LOW) and a TOGGLE button.

Clicking TOGGLE sends the command over serial and flips the local visual state immediately.

Every 200 ms the firmware broadcasts actual hardware FET states. The GUI overwrites all 6 indicators from this broadcast, making the display authoritative and self-correcting.

If firmware logic changes a FET internally, the GUI reflects the change within 200 ms without any additional protocol.

8.5  Connection and Disconnect Handling

The operator selects a COM port and baud rate from drop-downs, then clicks CONNECT. The background reader thread starts and the status indicator shows CONNECTED.

On disconnect — whether user-initiated or due to cable loss — all gauge needles return to zero and all FET indicators switch to red (LOW). This prevents any stale sensor reading from being misinterpreted as current live data.

8.6  Serial Log

Every incoming line that is not a FET state broadcast is appended to a scrolling log window.

A CLEAR LOG button clears the displayed log without affecting serial communication.

The log limits stored lines to a configurable maximum to prevent unbounded memory growth during long test sessions.

9.  Pending Features

The following features are designed and scoped but not yet implemented.

| Feature | Specification & Implementation Notes |
| --- | --- |
| SBUS RC Input (PA0) | Futaba SBUS protocol. Physical layer: inverted UART, 100,000 baud, 8E2 (even parity, 2 stop bits). Frame: 25 bytes, 11 servo channels × 11 bits each, plus flag byte. Signal is active-LOW (idle low). Requires hardware UART with inversion capability or a dedicated software UART. PA0 must be freed from RPM simulation before activation. |
| Real RPM — PB11 | Replace potentiometer simulation with actual pulse counting via external interrupt on PB11. Count rising edges over a fixed time window. RPM = (count ÷ pulses_per_revolution) × (60,000 ÷ window_ms). LM358 signal conditioner hardware is already installed and wired. |
| Engine State Machine | Five-state execution model: WAITING → STARTING → IDLING → OPERATING → COOLDOWN. Each state controls a specific combination of FET outputs. Transitions are triggered by RPM thresholds, EGT limits, and operator commands. Current state to be broadcast over serial for GUI display. |
| Over-temperature Protection | Automatic FET shutdown when TC1 (EGT) exceeds a configurable limit. Implemented as a firmware safety layer that cannot be overridden by GUI commands. Alert line sent over serial when triggered. |

10.  Known Constraints and Design Notes

10.1  Chip-to-Chip ADC Temperature Variation

The internal temperature sensor V25 constant varies up to ±0.15V between individual STM32F103 chips. If the MCU board is replaced, V25 must be recalibrated. Procedure: observe the reported MCU temperature at a known ambient temperature and adjust V25 until the reading matches. Each 10 mV change in V25 shifts the reading by approximately 2.3°C.

10.2  Shared GND Requirement

If the USB–UART adapter is not referenced to the same ground as the MCU board, the UART RX detection threshold will be incorrect and communication will produce garbage or no data. Always connect the GND pin of the adapter to the GND rail of the board, even when both are also connected to the same PC via USB.

10.3  PA0 Pin Conflict

PA0 is currently configured as an analog input for RPM simulation. When SBUS decoding is implemented on this same pin, the RPM test mode must first be moved to a different analog-capable pin.

10.4  MAX31855 Minimum Read Interval

The MAX31855 requires a minimum of 250 ms between successive SPI reads to complete its internal conversion. Polling faster returns stale data without any error flag. Do not reduce the thermocouple task interval below 250 ms.

10.5  ADC 12-Bit Initialisation

Most MCU frameworks default the ADC to 10-bit mode (0–1023). The firmware explicitly enables 12-bit mode (0–4095) in the startup sequence. If this initialisation call is absent or reset, all ADC-derived calculations will be incorrect by a factor of four.

10.6  FET Output Initialisation Order

All FET output pins must be set LOW as the very first action in the startup sequence, before any peripheral or sensor initialisation. If FET pins are configured after other setup code, they may float in an undefined state during the startup interval, potentially activating an actuator momentarily.

11.  Document Revision History

| Version | Date | Changes |
| --- | --- | --- |
| V1.0 | June 2026 | Initial firmware release — all implemented features documented in README |
| V2.0 | June 2026 | Restructured as implementation-independent specification. Added requirements table, circuit theory, ADC design rationale, GUI architecture, and pending features section. |