JET Engine ECU

Technical Specification  —  V1.0

NASTP  |  June 2026

1.  Project Goal

Design and build a custom Engine Control Unit (ECU) for monitoring and controlling a small JET engine during ground testing. The system must read engine sensors in real time, control engine actuators through FET outputs, and stream all data to a PC dashboard over a serial link. The operator must be able to monitor all channels and toggle actuators from the dashboard without any additional hardware.

2.  System Overview

The system has two parts that communicate over a single UART serial link:

•  ECU Board — Microcontroller + sensors + actuator drivers. Reads all inputs and drives all outputs.

•  PC Ground Station (GUI) — Python dashboard. Displays live sensor data and allows FET control from a desktop interface.

The serial link is bidirectional — the ECU streams sensor readings to the PC, and the PC sends FET toggle commands back to the ECU over the same wire.

3.  System Requirements

| # | Requirement | Detail |
| --- | --- | --- |
| R1 | Dual thermocouple reading | Read two K-type thermocouples. TC1 = exhaust gas (EGT). TC2 = intake/ambient. Handle open-circuit faults. |
| R2 | Battery voltage monitoring | Monitor 12V–24V battery. Scale to ADC range using resistor divider. Accuracy ±0.1V. |
| R3 | RPM measurement | Read engine RPM from a conditioned pulse signal. Range 0–120,000 RPM. |
| R4 | RPM simulation (test mode) | Simulate RPM via potentiometer input during bench testing without a running engine. |
| R5 | 6x FET output control | Drive 6 independent actuator channels (fuel valve, igniter, etc.). All OFF at power-up. |
| R6 | Bidirectional serial telemetry | Stream all readings to PC at 115,200 baud. Accept FET on/off commands from PC. |
| R7 | MCU temperature monitoring | Read microcontroller on-chip temperature sensor for thermal health monitoring. |
| R8 | Non-blocking firmware | All tasks run on independent timers. No channel may block another. |
| R9 | GUI ground station | Desktop dashboard showing all channels live. FET toggle buttons. Serial data log with clear function. |
| R10 | Real-time FET state sync | GUI must always show actual hardware FET state, not just last button press. |
| R11 | Safe disconnect | All GUI gauges reset to zero when serial link is lost. No stale readings displayed. |

4.  Hardware Components

| Component | Part | Purpose |
| --- | --- | --- |
| Microcontroller | STM32F103C8T6 (Blue Pill) | 72 MHz CPU, 12-bit ADC, hardware UART, 3.3V I/O, 64KB Flash |
| Thermocouple Amp #1 | MAX31855 | K-type thermocouple interface — exhaust gas temperature (EGT) |
| Thermocouple Amp #2 | MAX31855 | K-type thermocouple interface — intake / ambient temperature |
| Voltage Divider | 39 kΩ + 5.6 kΩ resistors | Scale battery voltage (up to 24V) down to MCU ADC input range (0–3.3V) |
| RPM Signal Conditioner | Dual LM358 op-amp | Clean and level-shift RPM sensor pulses to 0V/3.3V digital edges |
| RPM Test Buffer | Single LM358 (unity gain) | Isolate potentiometer output from MCU ADC input during RPM simulation |
| USB–UART Adapter | 3.3V TTL type | Connect MCU UART to PC USB for serial telemetry and commands |
| FET Driver Stage | Gate driver + MOSFET | Switch high-current engine actuators from 3.3V logic outputs |

5.  Pin Mapping

All I/O is 3.3V logic. Unallocated pins remain in default reset state.

| Function | Pin | Direction | Description |
| --- | --- | --- | --- |
| UART TX | PA2 | Output | Serial transmit — ECU telemetry to PC |
| UART RX | PA3 | Input | Serial receive — commands from PC |
| Battery Voltage | PA1 | Analog In | Voltage divider output (39K/5.6K). Max input ~24V |
| RPM Test Input | PA0 | Analog In | Op-amp buffered potentiometer — RPM simulation only |
| RPM Sensor | PB11 | Digital In | LM358-conditioned pulse train from engine RPM sensor |
| TC1 — SCK | PA5 | Output | MAX31855 #1 SPI clock (EGT / Exhaust) |
| TC1 — SO | PA6 | Input | MAX31855 #1 SPI data out |
| TC1 — CS | PB0 | Output | MAX31855 #1 chip select (active LOW) |
| TC2 — SCK | PB13 | Output | MAX31855 #2 SPI clock (Intake / Ambient) |
| TC2 — SO | PB14 | Input | MAX31855 #2 SPI data out |
| TC2 — CS | PB12 | Output | MAX31855 #2 chip select (active LOW) |
| FET Output 1 | PB6 | Output | Actuator/valve channel 1 — safe state LOW at power-up |
| FET Output 2 | PB5 | Output | Actuator/valve channel 2 — safe state LOW at power-up |
| FET Output 3 | PB9 | Output | Actuator/valve channel 3 — safe state LOW at power-up |
| FET Output 4 | PB8 | Output | Actuator/valve channel 4 — safe state LOW at power-up |
| FET Output 5 | PB4 | Output | Actuator/valve channel 5 — safe state LOW at power-up |
| FET Output 6 | PB7 | Output | Actuator/valve channel 6 — safe state LOW at power-up |
| MCU Temp Sensor | Internal | Analog In | On-chip die temperature — ADC channel 16, no external pin |

6.  Inputs and Outputs

6.1  Analog Inputs

| Signal | Pin | Range | Notes |
| --- | --- | --- | --- |
| Battery Voltage | PA1 | 0 – 3.3V (ADC) | Represents 0–24V battery via 39K/5.6K divider. 12-bit ADC, 64-sample average. |
| RPM Test Pot | PA0 | 0 – 3.3V (ADC) | Potentiometer via op-amp buffer. Mapped to 0–120,000 RPM for simulation. |

6.2  Digital / SPI Inputs

| Signal | Pins | Interface | Notes |
| --- | --- | --- | --- |
| TC1 — Exhaust Temp | PA5, PA6, PB0 | Software SPI | MAX31855. Range 0–900°C relevant. Returns NaN on open circuit. |
| TC2 — Intake Temp | PB13, PB14, PB12 | Software SPI | MAX31855. Range 0–100°C relevant. Same fault handling. |
| RPM Pulse | PB11 | Digital In | Conditioned pulse train. Interrupt counting for RPM calculation. (Pending implementation) |

6.3  Digital Outputs — FET Channels

| Channel | Pin | Safe State | Typical Use |
| --- | --- | --- | --- |
| FET 1 | PB6 | LOW | Engine actuator / fuel valve / relay |
| FET 2 | PB5 | LOW | Engine actuator / fuel valve / relay |
| FET 3 | PB9 | LOW | Engine actuator / igniter / relay |
| FET 4 | PB8 | LOW | Engine actuator / starter / relay |
| FET 5 | PB4 | LOW | Engine actuator / valve / relay |
| FET 6 | PB7 | LOW | Engine actuator / valve / relay |

6.4  Serial Interface (UART2)

| Direction | Format | Rate | Content |
| --- | --- | --- | --- |
| ECU → PC | ASCII text, \r\n terminated | Continuous | Sensor readings: temperature, voltage, RPM, MCU temp, FET states |
| PC → ECU | ASCII command, \n terminated | On demand | FET toggle commands: FETn:1 to turn ON, FETn:0 to turn OFF (n = 1–6) |

Baud rate: 115,200 bps  |  8N1  |  3.3V TTL levels  |  Shared GND required

7.  Sensor Update Rates

All tasks run independently on non-blocking timers. No task delays any other task.

| Measurement | Update Rate | Output Line Example |
| --- | --- | --- |
| Thermocouple TC1 + TC2 | Every 250 ms | TC1: 245.5 C ¦ TC2: 32.1 C |
| Battery Voltage | Every 500 ms | V_BATT: 24.01 V |
| Engine RPM | Every 100 ms | RPM: 17054 |
| MCU Die Temperature | Every 1000 ms | MCU: 24.4 C |
| FET State Broadcast | Every 200 ms | FET:1,0,0,1,0,0 |

8.  Pending Features

•  SBUS RC Input (PA0) — Futaba SBUS protocol — inverted UART at 100,000 baud, 8E2 framing, 11 channels. PA0 must be freed from RPM test mode first.

•  Real RPM (PB11) — Replace potentiometer simulation with interrupt-based pulse counting from the actual engine RPM sensor.

•  Over-temperature Protection — Automatic FET shutdown when EGT (TC1) exceeds a set limit. Independent of GUI commands.